ITEM MO-905 TOPSOILING

DESCRIPTION

905-1.1  This item shall consist of removing topsoil from areas to be stripped on the project site, preparing the ground surface for topsoil application, and placing and spreading the topsoil on prepared areas in accordance with this specification at the locations shown on the plans or as directed by the Engineer.

# MATERIALS

905-2.1  TOPSOIL.  Topsoil shall be a fertile, friable and loamy soil of uniform quality, without admixture of subsoil material, and shall be free from material such as hard clods, stiff clay, hardpan, partially disintegrated stone, pebbles larger than 2 inches in diameter and any other similar impurities. Topsoil shall be relatively free from grass, roots, weeds and other objectionable plant material or vegetative debris undesirable or harmful to plant life or which will prevent the formation of a suitable seedbed.

# CONSTRUCTION METHODS

905-3.1  GENERAL.  The areas to be stripped of topsoil and the stripping depths shall be shown on the plans.  Suitable equipment necessary for proper preparation and treatment of the ground surface, stripping of topsoil, and for handling and placing of all required materials shall be on hand, in good condition, and approved by the Engineer before the various operations are started.

905-3.2  OBTAINING TOPSOIL.  Prior to the stripping of topsoil from designated areas, any vegetation, briars, stumps and large roots, rubbish or stones found on such areas, which may interfere with subsequent operations, shall be removed using methods approved by the Engineer. Heavy sod or other cover, which cannot be incorporated into the topsoil by discing or other means, shall be removed.

The contractor shall remove all suitable topsoil from the designated areas on the project site and to the depth as directed by the Engineer. The topsoil shall be spread on areas already tilled and smooth-graded or stockpiled in areas approved by the Engineer. The quantity removed and placed directly or stockpiled shall be paid for at the contract unit price per cubic yard for “Class A Excavation” as specified in MO-152, Excavation and Embankment. When stockpiling of topsoil and later rehandling of such material is directed by the Engineer, the material so rehandled shall be paid for at the contract unit price per cubic yard for “Topsoiling” as provided herein.

905-3.3  PREPARING THE GROUND SURFACE.  Immediately prior to placing and spreading the topsoil on any area, the surface shall be loosened by discs or spike-tooth harrows, or by other means approved by the Engineer, to a minimum depth of 2 inches to facilitate bonding of the topsoil to the covered subgrade soil. The surface of the area to be topsoiled shall be cleared of all stones larger than 2 inches in any diameter and all litter or other material which may be detrimental to proper bonding, the rise of capillary moisture, or the proper growth of the desired planting. Limited areas, as shown on the plans, which are too compact to respond to these operations shall receive special scarification.

Grades on the area to be topsoiled, which have been established by others as shown on the plans, shall be maintained in a true and even condition. Where grades have not been established, the areas shall be smooth graded and the surface left at the prescribed grades in an even and properly compacted condition to prevent, insofar as practical, the formation of low places or pockets where water will stand.

905-3.4  PLACING TOPSOIL  The topsoil shall be evenly spread on the prepared areas to a uniform depth of 3 inches after compaction, unless otherwise shown on the plans or stated in the Special Provisions. Spreading shall not be done when the ground or topsoil is frozen, excessively wet, or otherwise in a condition detrimental to the work. Spreading shall be carried on so that turfing operations can proceed with a minimum of soil preparation or tilling.

After spreading, any large, stiff clods and hard lumps shall be broken with a pulverizer or by other effective means, and all stones or rocks (2 inches or more in diameter), roots, litter, or any foreign matter shall be raked up and disposed of by the Contractor. After spreading is completed, the topsoil shall be satisfactorily compacted by rolling with a cultipacker or by other means approved by the Engineer. The compacted topsoil surface shall conform to the required lines, grades, and cross sections. Any topsoil or other dirt falling upon pavements as a result of hauling or handling of topsoil shall be promptly removed.

## METHOD OF MEASUREMENT

905-4.1.  Topsoil stockpiled and rehandled by the contractor shall be measured by the number of cubic yards of topsoil measured in the stockpile. Topsoil shall be measured by volume in cubic yards computed by the method of end areas.

## BASIS OF PAYMENT

905-5.1.  Payment will be made at the contract unit price per cubic yard for Topsoiling. This price shall be full compensation for furnishing all materials and for all preparation, placing, and spreading the materials, and for all labor, equipment, tools, and incidentals necessary to complete the item.

Payment will be made under:

MO-905-5.1	Topsoiling--per cubic yard